Exercise 8.1.1 – 8.1.3

Exercise 8.2.1

If we enter a number beyond 0, let’s say we want to remove 1 when there are 0 matchsticks left. It stills works normally as the actual value in register shows -1, meanwhile the register shows a big number in unsigned decimal.

Exercise 8.2.2

- 0 < R2 < 4

- Two assembly instructions that can be used are: BGT and BLT

BGT: Z clear, N and V are the same.

BLT: N and V different

- If the first condition is not met (R2 > 0), and R2 is negative, then N = 1. If the first condition is not met (R2 > 0), and R2 = 0 then Z = 1

If the second condition is not met (R2 < 4), and R2 > 4 then C = 1. If the second condition is not met (R2 < 4), and R2 = 4 then both Z = 1 and C = 1.

- 1|      MOV R0, #15

2|      STR R0, .WriteUnsignedNum

3|      MOV R1, #msg1

4|      STR R1, .WriteString

5|Loop:

6|      MOV R1, #msg2

7|      STR R1, .WriteString

8|      LDR R2, .InputNum

9|start:

10|      CMP R2, #0

11|      BGT else1         //If R2 > 0 jump to else1

12|      B invalid1

13|else1:

14|      CMP R2, #4

15|      BLT cont          //If R2 < 4 jump to cont

16|      B invalid1

17|invalid1:

18|      MOV R1, #msg3

19|      STR R1, .WriteString

20|      LDR R2, .InputNum

21|      B start

22|cont:

23|      SUB R0,R0,R2

24|      STR R0, .WriteUnsignedNum

25|      MOV R1, #msg1

26|      STR R1, .WriteString

27|      B Loop

28|      HALT

29|msg1: .ASCIZ "remaining\n"

30|msg2: .ASCIZ "How many do you want to remove (1-3)?\n"

31|msg3: .ASCIZ "Please input a valid number\n"

Exercise 8.3.1

- LSL R4, R4, #30

LSR R4,R4, #30

- Select:

LDR R4, .Random

LSL R4,R4, #30

LSL R4,R4, #30

CMP R4, #0

BGT conti

B select

Conti:

STR R4, .WriteUnsignedNum

HALT

Exercise 8.3.2

MOV R0, #3

Select:

LDR R4, .Random

LSL R4, R4, #30

LSR R4,R4, #30

CMP R4, #0

BGT conti

B select

Conti:

CMP R4, R0

BGT select

B continue

Continue:

STR R4, .WriteUnsignedNum

HALT

Exercise 8.4.1 – 8.4.2

1|//Initialise matchstick count

2|      MOV R0, #15

3|Loop:

4|      STR R0, .WriteUnsignedNum

5|//Display initial message

6|      MOV R1, #msg1

7|      STR R1, .WriteString

8|//Computers Turn

9|select: LDR R2, .Random

10|      AND R2,R2, #3

11|      CMP R2, #0

12|      BEQ select

13|      CMP R2,R0

14|      BGT select

15|      BEQ select

16|cont: STR R2, .WriteUnsignedNum

17|      MOV R1, #msg4

18|      STR R1, .WriteString

19|      SUB R0, R0, R2

20|//Print remaining matchstick

21|      STR R0, .WriteUnsignedNum

22|      MOV R1, #msg1

23|      STR R1, .WriteString

24|//Check if computer wins

25|      cmp r0, #1

26|      BEQ computerWins

27|//Players turn

28|      MOV R1, #msg2

29|      STR R1, .WriteString

30|input: LDR R2, .InputNum

31|      CMP R2, #3

32|      BGT input

33|      CMP R2, #1

34|      BLT input

35|      CMP R2,R0

36|      BGT input

37|      SUB R0,R0,R2

38|      CMP R0,#1

39|      BEQ playerWins

40|      b Loop

41|playerWins: MOV R1, #msg3

42|      STR R1, .WriteString

43|      HALT

44|computerWins: MOV R1, #msg5

45|      STR R1, .WriteString

46|      HALT

47|msg1: .ASCIZ "remaining\n"

48|msg2: .ASCIZ "How many do you want to remove (1-3)?\n"

49|msg3: .ASCIZ "You Win!"

50|msg4: .ASCIZ "taken by computer\n"

51|msg5: .ASCIZ "Computer Wins!\n"